Meteorological System

| Revision | Date | Name | Comments |
| --- | --- | --- | --- |
| 1.0 | 22/12/2020 | Vladimir Trushin | First Revision |

The Weather Station is integrated by following sensors:

- Air temperature

- Relative humidity

- ADC

The Weather Station implemented using two boards of the Cortex-M4.

| Sensors | Quantity | Type |
| --- | --- | --- |
| Air temperature | 1 | I2C |
| Relative humidity | 1 | I2C |
| ADC | 1 | Digital |

| Resources of microcontroller | Quantity |
| --- | --- |
| UART | 2 |
| I2C | 1 |
| SPI | 2 |
| DAC | 1 |
| ADC | 1 |
| RTC | 1 |
| TIMER | 1 |

Communication between computer and microcontroller provides by UART.

The communication has two option:

- Main microcontroller to slave microcontroller  by NRF24L01+ and slave to PC by UART.

- Main microcontroller to PC by UART.

Block diagram

PC

Android

ADC

DAC

Sensor

Sensor

I2C

I2C

SPI

SPI

NRF

NRF

PC

PC

UART

UART

Cortex-M4 main

Cortex-M4 slave

TCP/IP - TCP

C++ application

Python application

Communication protocols:

| 4 bytes | 4 bytes | 4 bytes | 1 byte | 1 byte | 1 byte | 1 byte | 1 byte | 1 byte | 1 byte | 4 bytes | 4 bytes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Relative humidity | Air temperature | ADC | Year | Month | Day | Week day | Hour | Minute | Seconds | Prob time | Average |

Wireless communication with NRF24L01+ can be 1 kilometer.

Software on PC:

Implemented a base GUI Python 3 application on PC.

Probe time: the period time which changes the last update.

Average: number of probe-data during the period of time.

Humidity: percent of humidity.

Temperature: value in Celsius.

ADC Voltage: value in Volt.

Change time and date: must be a two-digit number for each part.

year: month:day:week day: hour: minute: seconds

for example: 20:12:22:3:16:32:30

Change time and average: seconds:average

1 <= seconds <= 1200            3 <= average <= seconds * 1000

Change port: can change the port.

- NRF24 – main to slave and slave to PC.

- UART – main to PC

Software on Android:

Implemented a base GUI C++ (QT) application on Android.

Probe time: the period time which changes the last update.

Average: number of probe-data during the period of time.

Humidity: percent of humidity.

Temperature: value in Celsius.

ADC Voltage: value in Volt.

T and D (Change time and date): must be a two-digit number for each part.

year: month:day:week day: hour: minute: seconds

for example: 20:12:22:3:16:32:30

T and A (Change time and average): seconds:average

1 <= seconds <= 1200            3 <= average <= seconds * 1000

IP and P (Enter IP and Port to connect): ip:port

for example: 192.168.1.12:12345